TPC-E Adjustment Calculation Examples

We have two Intel Xeon E5-2699 v4 processors in a two-socket server. An actual TPC-E submission for a Lenovo System x3650 M5 system has a score of 4,938.14.

What would the estimated TPC-E score be for a two-socket server that has two Intel Xeon E5-2667 v4 processors?

One method we can use (for processors in the same generation) is to adjust the actual TPC-E score by calculating the difference in the number of physical cores and the difference in the base clock speed between the two processors to come up with an estimated TPC-E score for a system with a particular processor.

In this example, the Intel Xeon E5-2699 v4 has 22 physical cores and a base clock speed of 2.2GHz, and the Intel Xeon E5-2667 v4 has 8 physical cores and a base clock speed of 3.2GHz.

Core count adjustment: 8/22 = .363

Base clock speed adjustment: 3.2/2.2 = 1.454

Calculation Example:

Actual Score		Core Count Adjust		Base Clock Adjust       	Estimated Score

4,938.14           times      	.363			times	1.454 		       = 	2,606.36

The two-socket Intel Xeon E5-2699 v4 system has an actual TPC-E score of 4,938.14, while the two-socket Intel Xeon E5-2667 v4 system has an estimated TPC-E score of 2,606.36. These numbers are a good way of measuring the total CPU capacity of a system.

Another useful calculation is to divide the actual or estimated TPC-E score by the total number of physical cores in the system.

The two-socket server with two Intel Xeon E5-2699 v4 processors has 44 total physical cores, while the two-socket server with two Intel Xeon E5-2667 v4 processors has 16 total physical cores.

Processor 	           Total Score	     	Total Physical Cores			Score/Core

Xeon E5-2699 v4           4,938.14			44					112.23

Xeon E5-2667 v4           2,606.36                              16					162.89

The score/core is a good measurement of the single-threaded performance of that processor.