TPC-E Adjustment Calculation Examples

There is an actual TPC-E score for a two-socket Lenovo ThinkSystem SR650 with two Intel Xeon Platinum 8280 processors with a score of 7,012.53.

What would the estimated TPC-E score be for a two-socket server that has two Intel Xeon Gold 6254 processors?

One method we can use (for processors in the same generation) is to adjust the actual TPC-E score by calculating the difference in the number of physical cores and the difference in the base clock speed between the two processors to come up with an estimated TPC-E score for a system with a particular processor.

In this example, the Intel Xeon Platinum 8280 has 28 physical cores and a base clock speed of 2.7GHz, and the Intel Xeon Gold 6254 has 18 physical cores and a base clock speed of 3.1GHz.

Core count adjustment: 18/28 = .643

Base clock speed adjustment: 3.1/2.7 = 1.15

Calculation Example:

Actual Score		Core Count Adjust		Base Clock Adjust       	Estimated score

7012.53           times      	.643			times	1.15		       = 	5,185.42

The two-socket Intel Xeon Platinum 8280 system has an actual TPC-E score of 7012.53, while the two-socket Intel Xeon Gold 6254 system has an estimated TPC-E score of 5,185.42. These numbers are a good way of measuring the total CPU capacity of a system.

Another useful calculation is to divide the actual or estimated TPC-E score by the total number of physical cores in the system.

The two-socket server with two Intel Xeon Platinum 8280 processors has 56 total physical cores, while the two-socket server with two Intel Xeon Gold 6254 processors has 36 total physical cores.

Processor 	           Total Score	     	Total Physical Cores			Score/Core

Xeon Platinum 8280    7,012.53   divided by		56				125.22

Xeon Gold 6254            5,185.42   divided by                       36				144.04

The score/core is a good measurement of the single-threaded performance of that processor.