Hardware Sizing Case Study #2

Use TPC-E OLTP benchmark results to help validate your hardware sizing estimates

Legacy System

HP Proliant DL580 G7 – Four-socket server, with four Intel Xeon X7560 processors, 1024GB of RAM

Actual raw TPC-E score is 2001.12 with 4 sockets, 32 physical cores, and 64 logical cores

2001.12/32 cores is 62.53/physical core

Processor details

2.27GHz base clock speed, 2.66GHz Turbo Boost, hyper-threading, NUMA, 24MB L3 cache, introduced Q1 2010

Actual TPC-E Comparison New System

IBM System x3650 M4 - Two-socket server, with two Intel Xeon E5-2697 v2 processors, 512GB of RAM

Actual raw TPE-E score is 2590.93 with 2 sockets, 24 physical cores, and 48 logical cores

2590.93/24 cores is 107.96/physical core (1.73 times higher single-threaded performance than existing system)

Processor details

2.7GHz base clock speed, 3.5GHz Turbo Boost, hyper-threading, NUMA, 30MB L3 cache, introduced Q3 2013

Adjusted TPC-E Recommended New System

Fujitsu Primergy RX500 S7 – Four-socket server, with four Intel Xeon E5-4650 processors, 512GB of RAM

Actual raw TPC-E score is 2651.27 with 4 sockets, 32 physical cores, and 64 logical cores

2651.27/32 cores is 82.85/physical core (1.32 times higher single-threaded performance than existing system)

Processor Details

2.7GHz base clock speed, 3.3GHz Turbo Boost, hyper-threading, NUMA, 20MB L3 cache, introduced Q2 2012

License Cost Savings

Licensing (24) SQL Server 2017 Enterprise Edition core licenses instead of (32) SQL Server 2017Enterprise Edition core licenses saves $57,024 in licensing costs ($6,874/core license)

| Sockets | CPU | Total Cores | TPC-E Score | Score/Core | License Cost |
| --- | --- | --- | --- | --- | --- |
| 4 | Xeon X7560 | 32 | 2001.12 | 62.53 | $228,096 |
| 4 | Xeon E5-4650 | 24 | 2651.27 | 82.85 | $171,072 |

Table 1: System Comparison Recap