Hardware Sizing Case Study #4

Use TPC-E OLTP benchmark results to help validate your hardware sizing estimates

Legacy System

Dell PowerEdge R900 – Four-socket server, with four Intel Xeon X7460 processors, 256GB of RAM

Actual raw TPC-E score is 671.35 with 4 sockets, 24 physical cores, and 24 logical cores

671.35/24 cores is 27.97/physical core

Processor details

2.66GHz base clock speed, no Turbo Boost or hyper-threading, 1066MHz front-side bus (SMP), 16MB L2 cache, introduced Q3 2008

Actual TPC-E Comparison New System

IBM System x3650 M5 - Two-socket server, with two Intel Xeon E5-2699 v4 processors, 512GB of RAM

Actual raw TPE-E score is 4938.14 with 2 sockets, 44 physical cores, and 88 logical cores

4938.14/44 cores is 112.23/physical core (4.01 times higher single-threaded performance than existing system)

Processor details

2.2GHz base clock speed, 3.6GHz Turbo Boost, hyper-threading, NUMA, 55MB L3 cache, introduced Q1 2016

Adjusted TPC-E Recommended New System

Dell PowerEdge R730 – Two socket server, with two Intel Xeon E5-2667 v4 processors, 512GB of RAM

Estimated raw TPC-E score is 2603.75 with 2 sockets, 16 physical cores, and 32 logical cores

2603.75/16 cores is 162.73/physical core (5.82 times higher single-threaded performance than existing system)

Processor Details

3.2GHz base clock speed, 3.6GHz Turbo Boost, hyper-threading, NUMA, 25MB L3 cache, introduced Q1 2016

TPC-E Score Adjustment Calculation

4938.14 divided by 2.75 (8 cores instead of 22 cores) = 1795.69

3.2GHz – 2.2GHz = 1000MHz. 1000MHz divided by 2.2GHz = 45.4% higher base clock speed

55MB L3 cache divided by 22 physical cores = 2.5MB cache per core (E5-2699 v4)

25MB L3 cache divided by 8 physical cores = 3.125MB cache per core (E5-2667 v4)

1795.69 times 1.454 = 2603.75 (due to higher base clock speed)

License Cost Savings

Licensing (16) SQL Server 2017 Enterprise Edition core licenses instead of (24) SQL Server 2017 Enterprise Edition core licenses saves $57,024 in licensing costs ($7,128/core license)

| Sockets | CPU | Total Cores | TPC-E Score | Score/Core | License Cost |
| --- | --- | --- | --- | --- | --- |
| 4 | Xeon X7460 | 24 | 671.35 | 27.97 | $171,072 |
| 2 | Xeon E5-2667 v4 | 16 | 2603.75 | 162.73 | $114,048 |

Table 1: System Comparison Recap